- Unmanned Aerial Vehicle Equipment and Flight Parameters

The DJI Mavic 3 Multispectral (DJI M3M) unmanned aerial vehicle used in this study: The UAV is equipped with a 20-megapixel visible light camera and four 5-megapixel multispectral sensors, enabling simultaneous acquisition of green, red, red-edge, and near-infrared bands. A built-in multispectral irradiance sensor records solar radiation in the image metadata, allowing illumination compensation during two-dimensional reconstruction and thus enhancing the accuracy and consistency of multi-temporal data. In addition, the DJI M3M, paired with an RTK module, provides centimeter-level positioning precision. The flight control, camera, and RTK module achieve microsecond-level synchronization, accurately obtaining the position information of the imaging center of each camera, enabling DJI M3M to perform image control-free aerial surveying even at high speeds. The core parameters of the aircraft are shown in Table 1.

Table 1

DJI M3M Aircraft Parameters.

| Category | Parameters | Values |
| --- | --- | --- |
| Aircraft | Max take-off weight | 1050g |
|  | Unfolded dimensions (L×W×H) | 347.5 mm×283 mm×139.6 mm |
|  | Max transmission distance | 32 km |
| RGB Camera | Sensor | 4/3 CMOS |
|  | Aperture | f/2.8–f/11 |
|  | Photo resolution | 5280× 3956 |
| Multispectral Camera | Sensor | 1/2.8 CMOS |
|  | Aperture | f/2.0 |
|  | Photo resolution | 2592×1944 |
|  | Band (Center±Full width at half maximum) | Green:560 nm±16 nm |
|  |  | Red:650 nm±16 nm |
|  |  | RedEdge:730 nm±16 nm |
|  |  | NIR:850 nm±26 nm |

- Flight Parameters

Flight altitude: 50 m.

Ground Sampling Distance (GSD): Approximately 0.88 cm/pixel for RGB and 3.1 cm/pixel for multispectral.

Overlap: Forward overlap and side overlap, for example, 80%.

Lighting conditions: Weather conditions during shooting (sunny/ few clouds) and time period (10:00 a.m. - 2:00 p.m.) to ensure consistent lighting.